TECHNICAL & PROCUREMENT
RECOMMENDATION

Single-Host Server PC
& Hospital Audio System

Radiology Queue Management System (RQMS)
Combined Military Hospital, Dhaka

ARCHITECTURE DECISION
One dedicated PC hosts the Angular frontend, FastAPI backend, database, WebSocket updates and cached announcement audio. It connects directly to the PA amplifier, so no separate Linux audio receiver application is required.

Prepared by Digi Drive Technology (DDT)

# 1. Recommended System Architecture

The radiographer uses an authorized browser on the hospital LAN. The dedicated server PC receives the call action, updates the queue and plays the announcement through its wired audio output.

RADIOGRAPHER PC
Authorized browser console · Call / Recall

↓  HOSPITAL LAN
HTTP(S) + WebSocket

DEDICATED SERVER PC
Angular UI · FastAPI · PostgreSQL · audio queue/cache

↓  USB SOUND CARD / 3.5 MM–RCA
Fixed wired line-level connection

PA AMPLIFIER
Existing hospital PA or dedicated 60W 70V/100V amplifier

↓  SPEAKER CABLE
70V/100V distributed circuit

WAITING-ROOM SPEAKERS
No sound from radiographer PCs, phones or displays

# 2. Why a Separate Receiver Is Not Required

The server PC is placed near the amplifier and keeps a permanent wired audio connection.

The backend already receives call/recall actions, so it can queue and play audio locally.

A serialized queue prevents overlapping announcements; generated WAV files are cached.

Browsers remain silent, preventing sound from radiographer computers and other client devices.

An operating-system startup unit can still auto-start the complete RQMS after reboot; this is not a separate receiver application.

# 3. Server PC Specification

MINIMUM
Intel N100/N97 · 8GB RAM · 256GB NVMe SSD · 1GbE wired LAN · USB audio or line-out · Ubuntu Linux LTS · UPS-backed.

RECOMMENDED
Intel Core i3 NUC class · 16GB RAM · 512GB NVMe SSD · wired LAN with static/reserved IP · external USB sound card · Ubuntu Linux LTS · separate backup destination.

The workload is modest, but extra memory and SSD capacity improve PostgreSQL headroom, report generation, backup retention and useful service life.

# 4. Bangladesh Market Survey

Market check completed on 26 July 2026. Prices and availability can change without notice. Obtain a formal quotation and confirm warranty, VAT, delivery and installation before purchase.

## 4.1 Server PC Options

RECOMMENDED · ASUS NUC 13 PRO CORE i3
Core i3-1315U · 8GB DDR4 · 256GB NVMe · Wi-Fi 6E · 2.5GbE. Listed at ৳57,000. Compact and low-power, with better service headroom. Upgrade to 16GB/512GB if the supplier can configure it.

View current Star Tech mini-PC listing

BUDGET · RYANS PC V100
Intel N100 · 8GB RAM · 256GB NVMe. Listed at ৳30,900. Sufficient for a small RQMS; request 16GB RAM and 512GB SSD where possible.

View Ryans PC V100

## 4.2 Audio and Power Components

USB AUDIO · DTECH DT-6006
Linux-compatible external sound card with 3.5 mm stereo output · Qty 1 · ৳450

Supplier listing

AUDIO CABLE
3 m, 3.5 mm male to 2×RCA male · Qty 1 · ৳340

Supplier listing

UPS · POWER GUARD PG 650VA PS
Metal body · 390W load capacity · Qty 1 · ৳4,300

Supplier listing

PA AMPLIFIER · CMX EA-60
60W · 70V/100V speaker output · RCA line input · Qty 1 · ৳21,960

Supplier listing

CEILING SPEAKER · CMX CSK-66E
Selectable 1.5/3/6W · 70V/100V · Suggested qty 4 · ৳4,080 each

Supplier listing

# 5. Indicative Procurement Packages

PACKAGE A · REUSE EXISTING CMH PA · ৳62,090
ASUS NUC i3 server + Dtech USB audio + RCA cable + metal-body 650VA UPS. Preferred where CMH already has a suitable amplifier line input and waiting-room speakers.

PACKAGE B · NEW FOUR-SPEAKER ZONE · ৳108,370
Package A + CMX 60W amplifier + four ceiling speakers + ৳8,000 provisional speaker cable/installation allowance. Use when radiology needs an independent audio zone.

PACKAGE C · BUDGET SERVER + EXISTING PA · ৳35,990
Ryans N100 server + Dtech USB audio + RCA cable + metal-body 650VA UPS. Lowest initial cost; upgrade RAM/storage where possible.

Exclusions: VAT/tax, monitor and keyboard for setup, network switch/cabling, rack/cabinet, civil works, surge protection, delivery and supplier installation unless quoted.

ELECTRICAL DESIGN CHECK
A 60W 100V amplifier supports four speakers tapped at 6W each (24W total) with useful headroom. A hospital PA/electrical technician must confirm speaker taps, cable gauge, zoning, grounding and amplifier input levels before installation.

# 6. Recommended Procurement Decision

Purchase the ASUS NUC 13 Pro i3 configuration, preferably upgraded to 16GB RAM and 512GB NVMe, together with the Dtech USB audio adapter and metal-body 650VA UPS. Connect it to the existing CMH PA amplifier.

If no suitable PA input exists, or radiology needs an independent zone, add the CMX EA-60 amplifier and four CMX CSK-66E ceiling speakers after an on-site acoustic and electrical survey.

# 7. Installation and Acceptance Checklist

Assign a static/reserved LAN IP and restrict access to authorized hospital subnets.

Install Ubuntu Linux LTS, PostgreSQL, espeak-ng and ALSA utilities.

Connect the USB sound card to an amplifier LINE/AUX input—never directly to a 70V/100V speaker circuit.

Fix the ALSA device identifier and verify it remains stable after reboot.

Set safe amplifier gain and verify clear speech across the waiting area.

Test call, recall, rapid calls, network interruption, application restart and power restoration.

Back up the database daily to a separate disk/NAS and perform a restore test.

Confirm all browsers remain silent and audio originates only from the server-connected PA.

# 8. Implemented Software Deployment

The RQMS production launcher now builds Angular and serves the frontend, REST API and WebSocket endpoints from one FastAPI process on port 8100. The backend announcement worker generates and caches WAV files, serializes playback, and sends audio to the server PC's configured ALSA/PulseAudio output.

A public tunnel is not required for hospital LAN operation. Ngrok remains optional for an authorized temporary demonstration only. Production access should follow CMH cybersecurity policy.

# 9. Key Risks and Controls

USB ADAPTER FAILURE
Keep one tested spare and pin the audio device in configuration.

POWER LOSS
Use a UPS, automatic application startup, graceful database shutdown and verified backup.

ANNOUNCEMENT OVERLAP
Use the backend's single serialized queue and bounded repeat count.

POOR INTELLIGIBILITY
Use wired 70V/100V distribution, test speaker tap levels and avoid Bluetooth.

SINGLE HOST FAILURE
Maintain configuration/database backup, a recovery guide and a spare SSD plan.

# 10. Source Register

Ryans PC V100 Intel N100 desktop

Star Tech portable mini-PC listing

Dtech DT-6006 USB audio converter

CMX EA-60 60W PA amplifier

CMX CSK-66E ceiling speaker

Power Guard PG 650VA PS UPS

Bangladesh audio cable listing